Quick Start Guide of DS-K1T502

Quick Start Guide of DS-K1T502

- Activate the Device via Mobile Phone

When the device is not activated, we can use the device AP hotspot connection to activate the DS-K1T502 device and connect to the device WEB for configuration operation.

- AP Connection: the initial password is the device serial number, for example: K49645302.

- Activate the device: Activating the device requires setting a password, and subsequent connections need to use this password to connect to the hotspot.

- Select three questions to answer and click Confirm. (If you forget the device activation password, you can change the password via security question.). Or you can choose E-mail method.

- Choose device language. Communication Settings: Privacy Wizard:

- Activate the Device via SADP

  - Run the SADP software and search the online devices. Find and select your device in online device list.

  - Input new password (admin password) and confirm the password. Click Active to start activation.

  - Status of the device becomes Active after successful activation.

  - Select three questions to answer and click Confirm. (If you forget the device activation password, you can change the password via security question.)

  - Change the device IP address to the same subnet as your computer by either modify the IP address manually or checking Enable DHCP.

  - Input the admin password and click Modify to activate your IP address modification.

  - Select the device and double-click the IP address to access the Web Client.

  - Enter the device user name and the password. Click Login.

- Add device to HikCentral Access Control

  - Enter HikCentral Access Control Web Client then choose ISAPI protocol and 80 port add device.

- Apply person to device

  - Create a name for the access level..

  - Click Add to add the access level and return to the access level management page.

  - Add person info on the platform such as card number, fingerprint, pin code etc.

  - On the top, select Access Control → Select Access Level → Manage Access Level on the left → Click Add to enter the Add Access Level page.

  - On the top, select Access Control → Select Access Level → Assign by Access Level on the left → Click on the access level that you want to assign to person.

  - On the assignee panel, click Assign To to show person list.

  - Select the persons whom you want to assign the access level to and click Add.

- Operation on device

  - On the top, select Device → Select Device and Server → Access Control Device on the left → Click  in the Operation column to enter the configuration page of a device.

  - Select Configuration on the left → Select Access Control → Select Authentication Settings

  - Chose Authentication method: card or fingerprint or password. Click Save.

  - Check the authentication event on the platform real-time monitoring.