How to Configure Video Recording on DS-K1T502

Preparation

- TF Card. Max. 256G supported

- DS-K1T502 Series

How to Configure Video Recording on DS-K1T502

- Install TF Card

  - Install TF Card and back cover before power on device. If you install TF card when device is powered on, please reboot device

- Configure Event Linkage

  - Log in device web page.

  - Go to configuration->Event->Event Detection-> Linkage Settings, click Add, then you can add new event and card linkage. Set trigger event according to your actual need, here we take door open event for example

  - Click General Linkage Settings to set recording time and recording delay. Then linkage setting is finished.

- Check recording video

  - Go to Event Search, you can select ‘View event with recorded video’ option to search for video.

  - Click the ‘play’ icon at the end of event , you can check recorded video