How to Use DS-K1T502 to Open Door via QR Code

Preparation

- DS-K1T502 device

- Computer running HikCentral AC or HikCentral Professional

https://www.hikvision.com/en/support/download/software/

How to Use DS-K1T502 to Open Door via QR Code

- Generate QR Code

  - Log in web page of HCAC, add device in Device module.

For DS-K1T502, choose Access Protocol as Hikvision ISAPI Protocol and Device port as 80

For other access control device, choose Hikvision Private Protocol and 8000 port

  - Go to Person module, add a person. Click ‘Credential Management’ to collect card number. If there is no real card, you can input a random number within 20 digits, for example: 12345678

  - After creating a person, you can check QR Code in person module. Click Download to download QR Code

- Set Access level

  - Go to Access module, set access level for K1T502, and assign access level to the person created in step 1, then software will apply person and card number to K1T502 device automatically.

- Enable QR Code Function of DS-K1T502

  - Enable QR Code function of DS-K1T502. Log in web page of DS-K1T502, go to Configuration->Access Control->Authentication Settings, Enable QR Code function

- Open door via QR Code

  - Prepare QR Code. Print QR Code or display QR Code on a screen with size no less than 6*6 cm. Put the QR Code in front of device camera within distance range from 13~25cm.